First general instructions:

- The following experiment is a MUltiple Stimuli with Hidden Reference and Anchor (MUSHRA) listening test.

- In this listening test, you will be asked to evaluate and assign grades to test signals that were processed with different spatial sound reproduction algorithms.

- In each screen you will listen to several test signals and compare them to a known reference signal.

- Rate the relative differences between the reference and the test signal on a scale from 0 to 100.

- The score 100 means that the signal is not differentiable from the reference. Values downwards towards 0 mean larger difference with respect to the reference.

- Differences can be either spatial artifacts, time varying artifacts or spectral artifacts.

- At least one signal should be rated 100, because a hidden reference signal is included as one of the signals to be graded.

- It is possible to give the same score to several signals.

- Please use the scale widely. Your grades should reflect your subjective judgement of the quality level for each of the presented test signals.

- You may listen to the signals in any order, any number of times.

- Please do not move your head while listening.

- First, a training phase will be performed

  - The training phase includes two parts: User interface training and signals familiarization.

  - The purpose of the training phase is to assist you, as an evaluator, to achieve the followings:

    - User Interface training: to learn how to use the test equipment and the grading scale; and

    - Signals familiarization: to become familiar with all the test signals and their quality level ranges.

  - After the training is done, the experiment will start.

  - The experiment includes XX screens with YY test signals.

- Adjust the volume level during the training and don't change it during the experiment.

Instruction for training - part 1:

Interface training:  Rate the relative differences between the following test signals and the reference. You must give 100 points to the hidden reference, and rank the other test signals in order that reflects the audible impairments.

Wrong rating messages:

Option 1: At least one of the test signals must be rated 100.

Option 2: Wrong rating! The hidden reference should be rated as 100. Press ok to try again.

Option 3: Wrong rating! You should rank the test signals in the correct order. Please listen carefully to the audible impairments. Press ok to try again.

Correct rating message:

That's right! This was an example of large and moderate differences.

Instruction for training - part 2:

Signals familiarization:

- These are the test signals that you will evaluate next.

- In this stage you will be presented with all test signals that you will evaluate in the listening test.

- The goal of this stage is to expose you to the full range and nature of impairments that you will experience during the test.

- You may take some time now to become familiar with the signals.

- Note that the signals are listed in a random order.

- You must listen at least once to each signal.